# Products checklist

## Main headline

JWC Honey Monster

## Images

Honey Monster Image

## Description

JWC’s Honey Monster® septage receiving station and Fats, Oils and Grease (FOG) receiving system quickly screens and processes septage, grease, FOG or sludge from haul trucks. This automated acceptance plant allows for cleaner handling of septage truck waste by reducing and separating unwanted solids such as rocks and rag. The unique combination of grinding, solids removal, washing and dewatering allows a typical septage truck to unload in 5 to 15 minutes. The septage receiving station is completely enclosed to ensure safety, vector control and containment of foul odours.

The automated Honey Monster septage receiving station makes managing septage waste easy for both the wastewater plant operator and the hauler.  The hauler just connects their cam lock hose and starts the flow.  Septage waste will pass by the SRS3000 Rock Trap before the Muffin Monster grinder shreds the debris found in septage waste.  Ultrasonic level sensors work with the system’s PLC to modulate the plug valve and regulate the flow into the screening area.  An auger screw and perforated screening trough screens out unwanted debris to protect downstream. Trash is washed, fully compacted and dewatered for disposal with JWC’s unique double helix compaction zone.

The Honey Monster is also available with the optional Monster Track system.  This option controls hauler access to the septage disposal systems and will provide data for accurate billing.  It provides unique identification of the hauler and tracks the quantity of FOG of septage waste discharged.  The Monster Track system can also monitor system levels and can alarm for unexpected pH levels in dumped waste.

## Features

- Hauler access control and billing with Monster Track system

- Dual Helix compaction zone for additional dewatering

- High level ultrasonic sensor controls inlet plug valve

- Triple spray wash manifold washes away soft organics

- Optional Heavy Object Trap (HOT) for pre-screening rocks and metal

## Applications

- Handling of septage truck waste

- Handling of FOG waste

## Downloads

Honey Monster Brochure

Please add these to the downloads folder.

## Video url link

None yet

## Tags / categories

I'll use the words tag and category interchangeably.

There are 4 types of categories:

Pump Type, Application, Industry, Brand .

I've put a list here. Just bold the ones that this product applies to.

| Pump Type | Application | Industry | Brand |
| --- | --- | --- | --- |
| Booster set Bore pumps Diaphragm pumps Dosing pumps Drum pumps Flexible impeller pumps Gear Pumps Macerators Lobe pumps Magnetic drive pumps Mixers Multistage pumps Odour filters Peristaltic hose pumps Piston pumps Progressive Cavity Pump Screw channel pumps Self priming pumps Submersible pumps Twin screw pumps Eccentric disc pumps | Desalination Dewatering Fire Floodwater Greywater Irrigation Sewage Sludge Stormwater Water Wastewater Food & Hygiene | Agriculture Aquaculture Building Environmental Industrial Mining Municipal Wastewater | Abaque Caprari Cornell Ebara Flygt Gorman-Rupp Grundfos Hidrostal Inoxpa JWC KSB Lowara Mono Netzsch Paco Remko Roto Southern Cross Spel Sulzer Tsurami Verderflex Waterco Wilden Zenit Zenox |

When we add products in the website administration area, we can categorise them using the list you provide above.